[[work]]

Modeling Assumptions (v0.1)

Project: TTFS UTRGV Cool Schools Date: June 2026

1. Remote Sensing Baseline

•	
      Surface Temperature (LST): Derived from Landsat 8/9 Thermal Infrared Sensor (TIRS) Band 10 data. Assumes emissivity based on NDVI.

•	
      Canopy Coverage: Derived using National Agriculture Imagery Program (NAIP) 1m resolution data paired with NDVI thresholding (NDVI > 0.4 indicates canopy).

•	
      Albedo: Estimated using Landsat surface reflectance.

2. Heat Reduction Forecast Model

•	
      Shade Impact: Every 10% increase in continuous canopy cover over impervious surfaces is modeled to reduce ambient surface temperature by approximately 1.5Â°C to 2.5Â°C during peak summer hours, assuming mature tree size (15-20 years post-planting).

•	
      Growth Rate: The model assumes an average canopy growth rate of 1.5 feet in diameter per year for the selected native species.

•	
      Evapotranspiration Cooling: Contributes an additional localized cooling effect of up to 1Â°C in areas with dense plantings and adequate soil moisture.

3. Dashboard Ingestion Logic

•	
      Uploaded CSV/GeoJSON files are assumed to be in WGS84 coordinate reference system (EPSG:4326).

•	
      Measurements lacking timestamps are assigned the ingestion date as a default baseline.

•	
      Missing temperature or canopy values are excluded from the aggregate averages to prevent downward skewing.